CONCRETE CANOE ENGINEERING CHEAT SHEET

Design A: 192" x 32" x 17" x 0.5" wall  |  174.26 lbs  |  60 PCF  |  f'r = 1500 psi  |  Crew: 700 lbs

1. HULL WEIGHT  —  How heavy is it?

girth = B + 2D    shell_area = girth x L x Cp    vol = area x t
weight = vol x density x 1.10   (Cp=0.55 taper, 1.10 overhead)

Design A: girth=5.50 ft, area=48.4 ft², vol=2.02 ft³, est=133 lbs. Actual = shell(163.11) + reinf(8.16) + finish(3.00)

= 174.26 lbs  (target <= 237 lbs)

2. DRAFT (Archimedes)  —  How deep does it sit?

V_disp = W_total / 62.4        (Archimedes: weight = displaced water)
A_wp = L x B x Cwp             (Cwp=0.70 canoe waterplane coefficient)
Draft = V_disp / A_wp

Design A: V=874.26/62.4=14.01 ft³, A_wp=16.0x2.667x0.70=29.87 ft², T=14.01/29.87=0.469 ft

= Draft = 5.63 in

3. FREEBOARD  —  How much hull above water?

Freeboard = Depth - Draft

Design A: F = 17.0 - 5.63

= Freeboard = 11.37 in    PASS (ASCE req >= 6.0")

4. STABILITY (GM)  —  Will it tip over?

GM = KB + BM - KG
KB = T/2    BM = I_wp/V_disp    I_wp = Cwp x L x B^3 / 12
KG = (W_hull x h_hull + W_crew x h_crew) / W_total

Design A: KB=2.81", BM=I_wp/V_disp=15.16", KG=(174.26x0.539+700x0.833)/874.26=0.774ft=9.29"

= GM = 2.81 + 15.16 - 9.29 = 8.68 in    PASS (ASCE req >= 6.0")

5. BENDING MOMENT  —  How much does it want to snap?

M_hull = (W/L) x L^2 / 8       (uniform distributed load, SS beam)
M_crew = P x L / 4              (point load at midspan)
M_total = M_hull + M_crew

Design A: w=174.26/16=10.89 lb/ft, M_hull=10.89x16²/8=348.5, M_crew=700x16/4=2800.0

= M_total = 3148.5 lb-ft

6. SECTION MODULUS  —  How strong is the cross-section?

U-shape: bottom plate (BxT) + 2 side walls (Tx(D-T))
y_NA = sum(A_i x y_i) / A_total    (neutral axis location)
I = sum(I_self + A x d^2)           (parallel axis theorem)
S = (I / c_max) x 0.75              (ACI thin-shell reduction)

Design A: A_bot=16.0, A_wall=8.25x2, y_NA=5.10", I=412.5 in^4, c_max=11.90"

= S = 58.00 in^3  (NOT bh^2/6=1541 -- that's solid rect, 27x too high!)

7. SAFETY FACTOR  —  Will it break?

sigma = M x 12 / S              (bending stress, convert lb-ft to lb-in)
SF = f'r / sigma                 (strength / stress)

Design A: sigma = 3148.5 x 12 / 58.00 = 651.45 psi, SF = 1500 / 651.45

= SF = 2.30    PASS (ASCE req >= 2.0)

ASCE 2026 COMPLIANCE SUMMARY

| Requirement | Threshold | Design A | Status |
| --- | --- | --- | --- |
| Freeboard | >= 6.0 in | 11.37 in | PASS |
| Metacentric Height (GM) | >= 6.0 in | 8.68 in | PASS |
| Safety Factor | >= 2.0 | 2.30 | PASS |
| Portland Cement | <= 40% of CM | 30.8% | PASS |
| Water/Cementitious | <= 0.50 | 0.36 | PASS |
| Reinforcement | <= 50% of wall | 0.34% | PASS |

References: Archimedes (287 BC) | Bouguer (1746) | ACI 318-19 | Beer & Johnston Mechanics of Materials | Hibbeler Structural Analysis | SNAME Principles of Naval Architecture