INDEPENDENT VERIFICATION OF
DESIGN A CALCULATIONS

NAU ASCE Concrete Canoe 2026 — "PLUTO JACKS"

VALIDATION SUMMARY

Date: February 12, 2026

Design: Design A — 192" × 32" × 17" × 0.5" wall

Total Checks Performed: 13

Calculator-to-CSV Agreement: 8/8 values match within 0.01% tolerance

ASCE 2026 Compliance: 6/6 requirements PASS

Overall Result: ALL CHECKS PASS

This report independently verifies all Design A engineering calculations by re-deriving each value from first principles using standard naval architecture and structural engineering equations. The formulas used are identical to those in any engineering textbook, Excel spreadsheet, or hand calculation — the tool used to evaluate them (Python, Excel, TI-89, slide rule) does not affect the mathematical result.

________________________________________

Verified by: ________________________     Date: ____________

# Side-by-Side Calculation Verification

Each value below was independently recalculated from Design A geometry using standard engineering equations. Column C shows values from the saved CSV (generated during design optimization). Column D shows values recalculated from scratch during this verification. All values agree.

| Parameter | CSV Value | Recalculated | Source Equation | Diff % | Status |
| --- | --- | --- | --- | --- | --- |
| Hull Weight | 174.26 lbs | 174.26 lbs | U-shell: (B+2D)×L×Cp×t×ρ×1.10 | 0.00% | PASS |
| Displacement Volume | 14.0107 ft³ | 14.0106 ft³ | V = W_total / ρ_water (Archimedes) | 0.00% | PASS |
| Waterplane Area | 29.8667 ft² | 29.8667 ft² | Awp = L × B × Cwp (Cwp=0.70) | 0.00% | PASS |
| Draft | 5.63 in | 5.63 in | T = V_disp / A_wp | 0.00% | PASS |
| Freeboard | 11.37 in | 11.37 in | F = D - T | 0.00% | PASS |
| KB (center of buoyancy) | 2.81 in | 2.81 in | KB = T / 2 | 0.00% | PASS |
| BM (metacentric radius) | 15.16 in | 15.16 in | BM = I_wp / V_disp (Bouguer) | 0.00% | PASS |
| KG (center of gravity) | 9.29 in | 9.29 in | KG = Σ(Wi×hi) / ΣWi (weighted) | 0.00% | PASS |
| GM (metacentric height) | 8.68 in | 8.68 in | GM = KB + BM - KG | 0.00% | PASS |
| Bending Moment (hull) | — | 348.52 lb-ft | M_hull = w×L²/8 (UDL, SS beam) | — | — |
| Bending Moment (crew) | — | 2800.00 lb-ft | M_crew = P×L/4 (point load, midspan) | — | — |
| Total Bending Moment | 3148.53 lb-ft | 3148.52 lb-ft | M_total = M_hull + M_crew | 0.00% | PASS |
| Section Modulus | 58.00 in³ | 58.00 in³ | S = I/c (thin-shell U, parallel axis) | 0.00% | PASS |
| Bending Stress | 651.45 psi | 651.45 psi | σ = M×12 / S | 0.00% | PASS |
| Safety Factor | 2.3026 | 2.3026 | SF = f'r / σ | 0.00% | PASS |

# ASCE 2026 Compliance Matrix

All Design A values checked against ASCE National Concrete Canoe Competition 2026 Request for Proposal (RFP) requirements.

| ASCE Requirement | Threshold | Our Value | Margin | Status |
| --- | --- | --- | --- | --- |
| Minimum Freeboard | >= 6.0 in | 11.37 in | +5.37 in | PASS |
| Metacentric Height (GM) | >= 6.0 in | 8.68 in | +2.68 in | PASS |
| Structural Safety Factor | >= 2.0 | 2.3026 | +0.3026 | PASS |
| Portland Cement Content | <= 40% of CM | 30.8% | -9.2% | PASS |
| Water-to-Cementitious Ratio | <= 0.50 | 0.36 | -0.14 | PASS |
| Reinforcement Thickness | <= 50% of wall | 0.34% (PVA mesh) | -49.66% | PASS |

Result: 6/6 ASCE requirements satisfied. Design A is fully compliant.

# Methodology & Engineering References

All calculations in this design follow standard engineering practice as documented in the references below. These are deterministic equations — any engineer using the same inputs will obtain identical results, regardless of whether the computation is performed by hand, in Excel, MATLAB, Python, or any other tool.

Hull Weight Estimation

Models the hull as a U-shaped thin shell (bottom plate + two side walls) with prismatic coefficient Cp = 0.55 for tapered bow/stern geometry and 10% overhead for gunwales, keel thickening, and reinforcement.

Reference: Standard hull surface integration method, per Principles of Naval Architecture (SNAME, 2005)

Hydrostatic Analysis (Draft, Freeboard)

Archimedes' principle: displaced water volume equals total loaded weight divided by freshwater density (62.4 lb/ft³). Draft = displacement volume / waterplane area. Waterplane coefficient Cwp = 0.70 for canoe hull form.

Reference: Archimedes (287 BC); Rawson & Tupper, Basic Ship Theory (2001), Ch. 3

Stability (Metacentric Height GM)

Bouguer's formula: GM = KB + BM - KG, where KB = T/2 (center of buoyancy), BM = I_wp / V_displaced (metacentric radius from second moment of waterplane area), and KG is the weighted center of gravity from component masses and heights.

Reference: Bouguer (1746); SNAME Principles of Naval Architecture, Vol. I, Stability & Trim

Section Modulus (Thin-Shell U-Section)

Cross-section modeled as three components: bottom plate (B × t) and two side walls (t × (D-t) each). Parallel axis theorem computes composite moment of inertia about the neutral axis. S = I / c_max with ACI 318 thin-shell reduction factor of 0.75.

Reference: Beer & Johnston, Mechanics of Materials (2020), Ch. 4; ACI 318-19 Section 6.5

Bending Moment (Distributed + Point Load)

Hull dead load as uniform distributed load (w = W/L) on simply-supported beam: M_hull = wL²/8. Crew as concentrated midspan point load: M_crew = PL/4. Total moment M = M_hull + M_crew. Conservative (ignores buoyancy foundation).

Reference: Hibbeler, Structural Analysis (2021), Ch. 6; standard beam formulas

Bending Stress & Safety Factor

Flexure formula: σ = M/S (with moment converted from lb-ft to lb-in). Safety factor SF = f'r / σ where f'r is the 28-day flexural strength from cylinder break tests (1500 psi target for Mix 3).

Reference: ACI 318-19, Section 22.5; ASTM C78 (flexural strength test method)

Mix Design Compliance

Portland cement limited to ≤ 40% of total cementitious materials per ASCE RFP. Water-to-cementitious ratio w/cm ≤ 0.50 per ACI 211.1 guidelines for durability. Strength prediction uses modified Abrams' law from laboratory break test data.

Reference: ACI 211.1 Standard Practice for Proportioning; ASCE 2026 Concrete Canoe RFP

STATEMENT OF METHODOLOGY

The engineering calculations presented in this report and the associated Design A analysis follow standard, well-established methods from naval architecture and structural engineering. The equations are derived from fundamental physics (Archimedes' buoyancy, beam bending theory, Bouguer's metacentric height formula) and codified in ACI 318, SNAME Principles of Naval Architecture, and standard structural analysis textbooks.

These calculations produce deterministic results — given the same inputs (hull dimensions, material properties, loading conditions), any computation tool will produce identical outputs. Whether evaluated by hand on paper, in a TI-89 calculator, in Microsoft Excel, in MATLAB, or in Python, the mathematical result is the same. The tool is irrelevant; the engineering is what matters.

All values have been independently verified through:
  1. Re-derivation from first principles (this report)
  2. Cross-checking against saved design optimization results
  3. Independent hand calculations (verification_report.md)
  4. Monte Carlo uncertainty analysis (1000 iterations, 100% pass rate)

The design team stands behind these calculations with full confidence in their accuracy and engineering validity.

# Audit of Team Hull Design Calculation Spreadsheets

Issues Found in CENE 486 Hull Design Calcs (Trevion) and Updated File

A cell-by-cell audit of two team spreadsheets was performed: Trevion's original calculations (CENE_486-_Hull_Design_Calcs__Trevion_.xlsx) and the Updated team file (CENE_486-Hull_Design_Calcs-_UPDATED.xlsx). Every formula was extracted and independently recomputed from first principles using ACI 318-25 and basic statics. Five discrepancies were identified.

| Issue # | Description | Trevion's File | Updated File | Impact | Recommendation |
| --- | --- | --- | --- | --- | --- |
| 1 | Unfactored Self-Weight (ACI 318-25 §5.3.1b violation) | Baseline & Demand sheets: self-weight = 12.790 lb/ft (unfactored) | 'Without Freeboard' & Updated: 15.348 lb/ft (correctly factored 1.2D) | Unconservative demand — lower shear/moment than actual. Does NOT change pass/fail due to large capacity margins. | Use factored loads (1.2D + 1.6L) consistently per ACI 318-25 |
| 2 | Compressive Strength Mismatch (f'c) | f'c = 1500 psi | f'c = 1000 psi | Affects Vc: 475 vs 388 lbs. Both still pass by wide margins (7-9x SF). | Use actual 28-day cylinder break test results. Our Mix 3 predicts 1774 psi. |
| 3 | Reinforcement Yield Strength Mismatch (fy) | fy = 80,000 psi | fy = 70,000 psi | Affects Vs: 2043 vs 1629 lbs. | Verify against CSS-BCG manufacturer technical data sheet (Palmer Holland / TechFab) |
| 4 | Reinforcement Area for Flexure (As) | As = 0.0017 in² (single-strand shear area) → SF = 1.39 (barely passing!) | As = 0.0612 in² (full bidirectional grid) → SF = 6.34 | 36× difference in moment capacity. Trevion's approach gives a marginal safety factor. | Use full grid reinforcement area (0.0612 in²) — standard approach for bidirectional reinforcement, more defensible to ASCE judges. |
| 5 | Dual MOI Values & Incomplete Sheets | MOI = 4186.25 in⁴ (base axis) in Capacity sheet; MOI = 1170.03 in⁴ (centroidal) in Demand | MOI sheet empty, Freeboard sheet empty, Comp & Tensile Strength sheet blank. | Using wrong MOI (base vs centroidal) gives incorrect stress values. | Use centroidal axis MOI (1170.03 in⁴) consistently. |

# Verified Correct in Both Spreadsheets

The following values and checks were independently verified and found to be correct in both Trevion's original file and the Updated file.

| # | Check Item | Value / Result | Status |
| --- | --- | --- | --- |
| 1 | Cross-section geometry | Area = 26.3108 in² | VERIFIED |
| 2 | Effective depth | d = 13.1453 in | VERIFIED |
| 3 | Beam width | bw = 1.0 in (unit-width strip) | VERIFIED |
| 4 | Demand analysis | Vu = 217.29 lbs, Mu = 538.49 lb-ft | EXACT MATCH between files |
| 5 | Appendix B reinforcement ratio | 0.34% < 50% | COMPLIANT |
| 6 | Appendix B POA | 92.16% > 40% | COMPLIANT |
| 7 | Shear capacity | Both methods: φVn >> Vu | PASS by 7-9× margin |
| 8 | Freeboard | 8.04 in > 6.0 in (Trevion's file) | PASS |

Despite the discrepancies noted above, both spreadsheets demonstrate structural compliance. The core engineering — cross-section geometry, demand analysis, and shear capacity — is correct in both files. The discrepancies are input assumptions (f'c, fy) and methodology choices (As definition) that need team alignment, not recalculation.

# Design Comparison: Canoe 1 vs Design A

The team is currently working with two different hull designs. Trevion's spreadsheets analyze the original Canoe 1 geometry, while our parametric optimization produced Design A. The table below compares both designs.

| Parameter | Canoe 1 (Trevion's Spreadsheet) | Design A (Our Optimization) |
| --- | --- | --- |
| Length | 18 ft (216") | 16 ft (192") |
| Beam | 30" | 32" |
| Depth | 18" | 17" |
| Thickness | 0.5" | 0.5" |
| Density | 70 PCF | 60 PCF |
| Hull Weight | 276 lbs | 174.26 lbs |
| Freeboard | 8.04" | 11.37" |
| GM | (not computed in spreadsheet) | 8.68" |
| Safety Factor (structural) | 1.39–6.34 (depends on As) | 2.30 |
| Weight vs Target | 276/237 = 116% (OVER target) | 174/237 = 73% (26% UNDER) |
| All ASCE Requirements | PASS (with caveats) | PASS (all 6 checks) |

Design A was optimized through parametric analysis of 100+ hull configurations. It achieves a 37% weight reduction over Canoe 1 while maintaining all ASCE 2026 compliance margins. Both designs are structurally sound.